CS 158A Final Exam Study Guide

Chapter 4:

IPv6:

0                    4		     12   16			24			  31

NEXT HEADER/DATA: Points to the header that is after the IPv6 packet.

Extension Header:

| Next Header | Reserved | Offset | RES | M |
| --- | --- | --- | --- | --- |

IDENT

MLPS: Multiprotocol Label Switching

Virtual circuit + Datagrams.

ADVANTAGES:

Relies on IP addresses and routing protocols to do its job like a datagram, and forward packets in relatively short, fixed length labels, with local scope like virtual network.

COST-EFFECTIVE: MPLS services can save a company between 10% to 25% compared to Frame Relay and ATM data services. If voice and data capabilities are added, the savings can jump up to approximately 40% for the entire network.

QOS CAPABILITIES: One main advantage of using MPLS services is its QoS capabilities. This is of upmost importance for companies utilizing voice and video.

PERFORMANCE UPGRADE: Increased response time and improved application performance are advantages for utilizing MPLS services. This reduces the number of “hops” between network points, which are the “any-to-any” features MPLS offers.

DISASTER RECOVERY: MPLS services offer data backup/disaster recovery capabilities by connecting data centers and other key sites in multiple redundant ways to the cloud. Unlike Frame Relay and ATM networks which require either switched or backup permanent-virtual-circuits, MPLS services allow remote sites to quickly and easily reconnect to backup locations, when applicable. This flexibility is a major advantage of MPLS over Frame Relay and ATM.

FUTURE-PROOFING THE NETWORK: Businesses realize MPLS is “the wave of the future” and are making the switch from Frame Relay and ATM services to MPLS. This switch is mainly for fear that they will be left behind. Competition is tight among businesses in all industries, so having a more cost-effective and more efficient network system gives them a fighting chance.

Mechanisms:

Enable IP capabilities on devices that do not have the capability to forward IP datagrams in the normal manner.

To forward IP packets along “explicit routes” precalculated that don’t necessarily match those match of IP routing protocols would select.

To support certain types of virtual private network services.

Label-based Forwarding:

ATM forwarding alg. + IP routing protocols with labels

| GFC | VPI | VCI | PTI | CLP | HEC | DATA |
| --- | --- | --- | --- | --- | --- | --- |

^                 ^                         ^

LABEL

“Shim” header (for PPP, Ethernet, etc.)

| PPP Header (Layer 2) | LABEL HEADER | Layer 3 Header |
| --- | --- | --- |

The nodes forward their labels to their neighboring nodes.

Explicit Routing:

| Label (Incoming) | Input | Output | Remote Label (Outgoing) |
| --- | --- | --- | --- |

Constrained Shortest Path First: constrained the link to available capacity

Layer 2 Tunneling:

MPLS header added instead of IP header with IP Tunneling.

Stacking Layer 2 with the head adding a DEMUX label (virtual circuit emulator) and Tunnel label (follows the correct path to tail router)

An ATM cell arrives on the destgnated input port with the appropriate VCI value.

The head router attaches the demultiplexing label that identifies the emulated circuit.

The head router then attaches a second label, which is the tunneling label.

Routers between the head and the tail forward the packet using only the tunnel label.

The tail router removes the tunneling label, finds the demux label and recognizes circuit.

The tail router modifies the ATM VCI to the correct value and sends it out to the correct port.

Layer 3 VPN: profvider = MPLS enabled: Label swapping (VC) + Control routing protocol (datagram)

IP packets

Chapter 5:

UDP Header:

TCP Header:

Functions:

tcp = recv, send
    udp = recvfrom,sendto

TCP Handshaking:

To establish a connection, TCP uses a three-way handshake. Before a client attempts to connect with a server, the server must first bind to a port to open it up for connections: this is called a passive open. Once the passive open is established, a client may initiate an active open. To establish a connection, the three-way (or 3-step) handshake occurs:

SYN: The active open is performed by the client sending a SYN to the server. It sets the segment's sequence number to a random value A.

SYN-ACK: In response, the server replies with a SYN-ACK. The acknowledgment number is set to one more than the received sequence number (A + 1), and the sequence number that the server chooses for the packet is another random number, B.

ACK: Finally, the client sends an ACK back to the server. The sequence number is set to the received acknowledgement value i.e. A + 1, and the acknowledgement number is set to one more than the received sequence number i.e. B + 1.

At this point, both the client and server have received an acknowledgment of the connection.

TCP Sliding Window Alg:

Used for Flow Control and SeqNum <= (MaxSeqNum +1)/2

LastByteAcked and LastByteSent represent the sliding window of the sending app

The receiving app uses NextByteExpected and LastByteRcvd

They both have a buffer, MaxSendBuffer and MaxRcvBuffer.

Advertised Window = MaxRcvBuffer – (NextByteExpected-1) – LastByteRead

Where LastByteRead < NextByteExpected

LastByteSent – LastByteAcked <= Advertised Window

EffectiveWindow = Advertised Window – (LastByteSent – LastByteAcked)

LastByteWritten – LastByteAcked <= MaxSendBuffer.

Wraparound:

TCP’s Sequence Number = 32 bits; AdvertisedWindow = 16 bits

With MSL, make sure that no sequence number can wrap around within the 120 second period of time.

For OC the 32 bits aren’t enough for the wraparound time.

Keeping the Pipe Full:

Delay x Bandwidth can indicate the AdvertiseWindow because the receivier can not open it as large and still have enough buffer space.

100 ms of RTT = typical number for a cross-country connection in which the delay x bandwidth product is the AdvertisedWindow size.

Adaptive Retransmission:

Reasons: ACK lost and early transmit = duplicate packets, and then there is packet loss

Too short -> wasted retransmissions and duplicates

Too long -> excessive delays when packet is lost

Estimated RTT = a * CurrentEstimatedRTT + (1-a)*SampleRTT

Compute timeout = 2 * EstimatedRTT where 0.8 <= a <= 0.9

TCP Extensions:

1st option: TCP’s timeout mechanism – subtracting timestamp

2nd option: TCP’s 32-bit SequenceNum – add timestamp so you know which sequence number is which

3rd option: Advertise a larger window – defines a scaling factor

4th option: Selective Acknowledgements – in the header of the ack, optional fields tell if additional blocks

Chapter 6:

TCP Congestion Control:

AIMD (Additive Increase and Multiplicative Decrease): For every segment that makes a RTT is back, the window size will increase by 1. When a loss is detected, the congestion window size will be reduced by half.

Slow Start: Starts the congestion window at 1, increase the window size exponentially (2n) each time an acknowledgement is received, but when the acknowledgement is not received for the segment or hits the predetermined threshold value, congested window size goes down to 1 and the new threshold for the congested window goes down by half.

Fast retransmission: If a TCP sender receives three duplicate acknowledgements with the same acknowledge number (that is, a total of four acknowledgements with the same acknowledgement number), the sender can be reasonably confident that the segment with the next higher sequence number was dropped and then can retransmit without waiting for a timeout.

Fast Recovery: Like slow start but instead of starting from 1, start from the new congestion window threshold and grow linearly.

Difference between AIMD, Slow Start, and Fast recovery:

Refer above?